Permanent Magnet DC  Brushed and Brushless  Motors

Justin Beemer, Michael Reber, Adithya Subbiah, Karthik Urs EECS 373 F19

Background Information

In a magnetic field (B), magnetic objects want to align their  own field with the lines of the magnetic field.

See: Compass

Background Information

In the compass example, both the grey bar magnet and the  compass needle have their own fields.

If you turn the bar magnet, the needle will follow in order to  align its field with the bar’s field.

The needle turns because there is a torque induced by the  misalignment of magnetic fields.

You can also create a magnetic field using a coil of conductive  material...

Background Information

In the compass example, both the grey bar magnet and the  compass needle have their own fields.

If you turn the bar magnet, the needle will follow in order to  align its field with the bar’s field.

The needle turns because there is a torque induced by the  misalignment of magnetic fields.

You can also create a magnetic field using a coil of conductive  material...

Background Information

In the compass example, both the grey bar magnet and the  compass needle have their own fields.

If you turn the bar magnet, the needle will follow in order to  align its field with the bar’s field.

The needle turns because there is a torque induced by the  misalignment of magnetic fields.

You can also create a magnetic field using a coil of conductive  material...

Background Information

In the compass example, both the grey bar magnet and the  compass needle have their own fields.

If you turn the bar magnet, the needle will follow in order to  align its field with the bar’s field.

The needle turns because there is a torque induced by the  misalignment of magnetic fields.

You can also create a magnetic field using a coil of conductive  material...

Background Information

In the compass example, both the grey bar magnet and the  compass needle have their own fields.

If you turn the bar magnet, the needle will follow in order to  align its field with the bar’s field.

The needle turns because there is a torque induced by the  misalignment of magnetic fields.

You can also create a magnetic field using a coil of conductive  material...

Commutation

Recall the equation for torque on a current loop in a magnetic field: T = IA ⨉ B

where T is torque, I is current flowing through the loop, A is the area vector of the  current loop, and B is the magnetic field vector.

In an electric motor, current flows through a loop/coil on an armature. If you  control B or A to maintain angular separation, there will be a constant torque and  the armature will spin continuously.

Keeping these fields angularly separated is a process known as Commutation.

Brushed DC Motors

Brushed DC (BDC) motors are the category of motors that achieve  commutation using Brushes -- electrical conductors that mate with other  conductors in a sliding interface. It’s easier to understand graphically:

Brushed DC Motors

In the figure, the brushes are fixed (attached  to the Stator, along with a set of permanent  magnets) and they electrically mate with the  Commutator in order to change how current

flows in the coils that sit on the armature. The  entire rotating assembly is called the Rotor,  which can be attached to a mechanical load  to be moved.

Remember: with magnet(ic field)s, opposites  attract!

Brushed DC Motors

In the figure, the brushes are fixed (attached  to the Stator, along with a set of permanent  magnets) and they electrically mate with the  Commutator in order to change how current

flows in the coils that sit on the armature. The  entire rotating assembly is called the Rotor,  which can be attached to a mechanical load  to be moved.

Remember: with magnet(ic field)s, opposites  attract!

Brushed DC Motors

In the figure, the brushes are fixed (attached  to the Stator, along with a set of permanent  magnets) and they electrically mate with the  Commutator in order to change how current

flows in the coils that sit on the armature. The  entire rotating assembly is called the Rotor,  which can be attached to a mechanical load  to be moved.

Remember: with magnet(ic field)s, opposites  attract!

Brushed DC Motors

In the figure, the brushes are fixed (attached  to the Stator, along with a set of permanent  magnets) and they electrically mate with the  Commutator in order to change how current

flows in the coils that sit on the armature. The  entire rotating assembly is called the Rotor,  which can be attached to a mechanical load  to be moved.

Remember: with magnet(ic field)s, opposites  attract!

How to Use BDC Motors

To control a brushed DC motor, you  need hardware for a Power stage between your processor and the  terminals of the motor.

Two half-bridges can be combined to  form an H-bridge, which when  combined with your power supply and a  PWM signal can control a brushed DC  motor speed and direction.

How to Use BDC Motors

If you turn on transistors Q1 and Q3,  current will flow across the DC Motor in  one direction, turning it via the process  described earlier.

By turning on transistors Q2 and Q4, the  current flows in the reverse direction and  spins the motor in reverse.

By controlling the duty cycle of the PWM  wave applied to the transistors, you change  the apparent voltage across the motor and  thus the motor speed.

How to Use BDC Motors

If you turn on transistors Q1 and Q3, current  will flow across the DC Motor in one  direction, turning it via the process described  earlier.

By turning on transistors Q2 and Q4, the  current flows in the reverse direction and  spins the motor in reverse.

By controlling the duty cycle of the PWM  wave applied to the transistors, you change  the apparent voltage across the motor and  thus the motor speed.

How to Use BDC Motors

If you turn on transistors Q1 and Q3, current  will flow across the DC Motor in one  direction, turning it via the process described  earlier.

By turning on transistors Q2 and Q4, the  current flows in the reverse direction and  spins the motor in reverse.

By controlling the duty cycle of the PWM  wave applied to the transistors, you  change the current through the motor and  thus the motor speed.

How to Use BDC Motors

If you turn on transistors Q1 and Q3, current  will flow across the DC Motor in one  direction, turning it via the process described  earlier.

By turning on transistors Q2 and Q4, the  current flows in the reverse direction and  spins the motor in reverse.

By controlling the duty cycle of the PWM  wave applied to the transistors, you  change the current through the motor and  thus the motor speed.

How to Use BDC Motors

If you turn on transistors Q1 and Q3, current  will flow across the DC Motor in one  direction, turning it via the process described  earlier.

By turning on transistors Q2 and Q4, the  current flows in the reverse direction and  spins the motor in reverse.

By controlling the duty cycle of the PWM  wave applied to the transistors, you  change the current through the motor and  thus the motor speed.

Brushed DC Motors

Pros:

● Inexpensive ● Easy to control ● Simple to model

Cons:

● Large

● Heavy

● Wear from  brushes

● Linear Torque  Speed curve

Applications:

● Old electric

vehicles

● DIY

● Low cost, low  power applications

Brushless DC Motors

Brushless DC (BLDC) motors don’t have brushes to  handle commutation -- it must be done

electronically!

The current through the coils is commutated  electrically order to generate torque to spin on the  rotor.

The current on three inputs (known as Phases) can  be controlled using solid state power electronics  (MOSFETs, IGBTs, BJTs).

How to Drive Brushless Motors

● Add another half bridge (three

phases)

● Each half bridge controls a single

phase

● Transistors switch on and off (PWM)

to modulate current

● Capacitor and motor act and low

pass filter.

○ PWM -> Sine wave

Brushless DC Motors: Control Strategies

Brushless motors require electrical commutation that can be achieved with a  variety if control strategies.

Trapezoidal Control Field-Oriented Control (FOC)

Current

Vector

Brushless DC Motors: Control Strategies

Trapezoidal Control

● Uses 3 digital sensor with 60 degree  resolution

● Commutates the phases to maintain a B lead of 60-120 degrees (before jumping to  the next state)

● This variable lead angle can cause  Torque Ripple.

● The algorithm is simple, but does not  provide optimal performance

Field-Oriented Control (Vector Control)

● Uses a high-resolution rotary position  sensor

● Samples phase currents to calculate and  control current vector.

● Runs a feedback control loop to maintain a  specified B lead current vector by 90  degrees

● Complex algorithm and hardware, but  higher performance

Electronic Speed Controllers (ESCs)

DC Input

Three Phase Output

Hall effect sensor input

AC Inverters

Tractive Inverter from RMS

Brushless DC Motors

Pros:

● High power

density

● Non linear torque  speed curve

● No wear from  brushes

● Efficient

Cons:

● Expensive

● Hard to control

Applications:

● RC products ● Modern Electric  Vehicles

● Industrial

machines and

drives

Basic Analysis for Motor Sizing (BDC)

Avoid thinking of your problem in

terms of speed.

It takes torque to move things. Power is

how fast you can spin while applying that

torque.

Power = Torque ✕ Speed

Think of these two parameters first!

Basic Analysis for Motor Sizing (BDC)

1. Find the location of your application

torque

2. Trace upwards and take note of the

intersections with the speed, power,

efficiency, and current curves

S

3. Evaluate if the values at those

t

e

p

intersections meet your criteria

2

Step 1

Basic Analysis for Motor Sizing (BDC)

Not fast enough?

Not efficient enough?

Can’t sustain current draw?

● Get a bigger motor OR

● Gear your motor towards peak power

(if to the right -> gear down and vice

Gear Down ~2:1

S

t

e

p

versa)

2

● (or gear towards peak efficiency)

Ex: Gear down 2:1 -- Torque from motor

is halved; speed more than doubles;

efficiency increases (motor runs cooler)Step 1

Selection Guide

Numbers are a relative scale -- higher is better

Property BDC BLDC - Trapezoidal BLDC - FOC Simplicity/Affordability 3 2 1 Power/$ at low power 3 2 1 Power/$ at high power 1 2 3

Power/Mass 1 2 3 Speed 1 2 2 Torque at low power 3 1 2 Torque at high power 1 2 3

Questions?

Appendices

PMSM Induction Motors Switched Reluctance