Beaglebone pinout

Stepper Motor Driver:

Store page https://www.pololu.com/product/2134

Datasheet https://www.pololu.com/file/0J617/drv8834.pdf

Stepper Motor

Datasheet https://www.pololu.com/file/0J635/SY20STH30-0604A.pdf

Wiring stepper motor to driver board:

B2 B1 A1 A2 -> black green red blue

The order doesn’t seem to matter as long as black and green are together and red and blue are together

Wiring Stepper 1:

STEP - P8_7

DIR - P8_9

SLEEP - SYS_5V

VMOT - power supply V_in

Both GNDs - GND

Wiring Stepper 2:

STEP - P8_8

DIR - P8_10

SLEEP - SYS_5V

VMOT - power supply V_in

Both GNDs - GND

stepper.py rotates the stepper motors by user input degrees

Located in the electrical team’s beaglebone /examples/extras/stepper.py and in this drive folder

stepper_position.py rotates the stepper motors to user input degree position

Located in the electrical team’s beaglebone /examples/extras/stepper_position.py and in this drive folder

stepper_back_forth.js rotates the stepper motor back and forth

These files are also in the Beaglebone, located in the examples folder

For 17HS16-2004S1, we used stepper 1 connections and 3V